Journal of National Science Foundation, 38 4 (2010) 213-218

___________________________________________________________________________________________________________________

Simulation of Diffusion Limited Aggregation in Field Programmable Gate Arrays

W.A.S Wijesinghe1, M.K Jayananda2 and D.U.J Sonnadara2

1Department of Electronics, Wayamba University of Sri Lanka, Sri Lanka

2Department of Physics, University of Colombo, Sri Lanka

ABSTRACT

This paper presents design considerations and performance in implementation of Diffusion-limited aggregation (DLA) process on a Xilinx Spartan 3 Field Programmable Gate Array (FPGA). The DLA cluster algorithm was implemented as a block RAM and two 32-bit Linear Feedback Shift Register random number generators in hardware. The complete design, written in VHDL and synthesized using Xilinx WebPACK 7.2 was downloaded to the Spartan 3 device for speed measurements.  A 300% speed improvement compared to a software based implementation of the same algorithm was observed when the design was tested in a XC3S1000 FPGA operated with a 100 MHz clock.